EXPERIMENT 9:

SOURCE CODE:

// Program Sketch

int trig = 10;

int echo = 9;

long duration;

int cm;

void setup() {

// put your setup code here, to run once:

pinMode(trig, OUTPUT);

pinMode(echo,INPUT);

Serial.begin(9600);

}

void loop() { // Sketch to calculate ultrasonic distance

digitalWrite(trig, LOW);

delayMicroseconds(10);

digitalWrite(trig, HIGH);

delayMicroseconds(10);

digitalWrite(trig, LOW);

delayMicroseconds(10);

duration = pulseIn(echo, HIGH);

cm = (duration/2) * 0.034;

// Print Value on Serial Monitor

Serial.print(" Distance = ");

Serial.print(cm);

Serial.println(" cm");

}