EXPERIMENT 18:

SOURCE CODE:

#include <reg51.h>

#include <stdio.h>

// Function declarations

void delay(void);

// Serial Interrupt Service Routine

void serial_ISR(void) interrupt 4 {

char ch;

// Check if interrupt was triggered by reception

if (RI) {

RI = 0;          // Clear receive interrupt flag

ch = SBUF;       // Read received character

// Print character safely (avoids mixing with printf in main)

putchar('\n');

putchar('>');

putchar('>');

putchar('>');

putchar(' ');

putchar(ch);

putchar('\n');

}

}

// Visual delay loop

void delay(void) {

unsigned int i;

for(i = 0; i < 50000; i++);

}

void main(void) {

// Hardware Initialization

SCON = 0x50;     // UART mode 1 (8-bit, variable baud), REN=1

TMOD = 0x20;     // Timer 1, mode 2 (8-bit auto-reload)

TH1  = 0xFD;     // 9600 Baud rate (at 11.0592 MHz crystal)

TR1  = 1;        // Start Timer 1

// Interrupt Configuration

IE   = 0x90;     // Enable global interrupts (EA=1) and Serial interrupt (ES=1)

TI   = 1;        // Set TI to 1 to kickstart printf

// Main execution loop

while(1) {

printf("Hello World\n");

delay();

}

}